ZTLP

Zero Trust Layer Protocol

Draft RFC – Version 0.5  |  Experimental / Informational

─────────────────────────────────────────

Author: Steven Price

ZTLP.org

Monrovia, CA

Status: Internet-Draft – This document is intended for public discussion. Distribution is unlimited.

This document is not an IETF RFC. It is a working concept specification.

2026

# Abstract

Distributed Denial of Service (DDoS) attacks and unauthenticated network access represent the two most persistent and costly structural failures of the modern Internet. Both exist for the same reason: the Internet was designed to carry any packet from any source, with security left as an afterthought for endpoints and applications to solve individually. ZTLP (Zero Trust Layer Protocol) is a direct response to this design flaw.

ZTLP defines a secure, identity-first network overlay that enforces cryptographic authentication before any network state is allocated. Unauthenticated packets are silently dropped at the edge — no CPU consumed, no memory allocated, no state created. This makes volumetric DDoS attacks structurally ineffective and eliminates the concept of an open attack surface entirely. ZTLP rides over existing IPv4 and IPv6 infrastructure and is invisible and unusable to any node that does not possess a valid, hardware-backed cryptographic identity.

The primary design goals of ZTLP are DDoS mitigation and Zero Trust network access at the protocol layer — not at the application, firewall, or middleware layer where such controls are currently implemented. Bandwidth reservation across the public Internet backbone is a future target capability of ZTLP, motivated by its application as a structural DDoS defense mechanism. This document is submitted as an Informational/Experimental specification. It does not propose changes to IP addressing or BGP routing. It defines a protocol layer that can be deployed today over the existing Internet.

# Table of Contents

1.  Problem Statement

2.  Introduction and Motivation

3.  Terminology

4.  Goals, Non-Goals, and Design Philosophy

5.  Threat Model

6.  Protocol Overview

6.  ZTLP Addressing – Node Identity

7.  ZTLP Packet Format

8.  ZTLP-NS – Distributed Trust Namespace

9.  Node Initialization and Bootstrap Procedure

10. Handshake and Session Establishment

11. Relay Node Architecture

12. Hardware Enforcement Profiles

13. Transport Fallback and NAT Traversal

14. Routing and Path Selection

15. Key Management and Hardware Identity

16. Deployment Model and Migration Strategy

17. Security Considerations

18. Privacy Considerations

19. Operational Tooling

20. Open Issues and Future Work

21. References

# 1.  Problem Statement

The Internet was not designed to be secure. It was designed to be resilient and reachable — to move packets from one place to another regardless of who sent them or why. That was the right decision in 1974. It is the source of most of our problems in 2026.

## 1.1  The Cost of the Status Quo

The structural insecurity of the Internet is not an abstract concern. It has measurable, recurring costs that affect every organization connected to it:

- DDoS attacks cost the global economy an estimated $2.5 billion annually in direct mitigation costs, downtime, and lost revenue. Volumetric attacks regularly exceed 1 Tbps. They work because the Internet accepts any packet from anyone.

- Ransomware attacks on critical infrastructure — hospitals, water treatment facilities, school districts, manufacturers — succeed primarily because internal networks are flat and unauthenticated. Once an attacker is inside, lateral movement is trivial. The network has no concept of identity; if you can reach an IP, you can attempt to exploit it.

- BGP route hijacking allows nation-state actors and misconfigured routers to silently redirect Internet traffic, intercept communications, or cause widespread outages. RPKI deployment has improved but remains incomplete, and BGP itself has no cryptographic path validation.

- IoT insecurity is systemic and worsening. Billions of devices connect to the Internet with no meaningful authentication, becoming permanent botnet members and attack platforms. The network layer offers them no protection and demands nothing of them.

- Compliance frameworks are compensating controls, not solutions. HIPAA, PCI-DSS, CMMC, and SOC 2 require organizations to build security on top of an insecure network layer using firewalls, VPNs, intrusion detection, and monitoring. These are necessary but they are expensive, operationally complex, and fundamentally reactive. They detect and respond to threats that a secure-by-design network layer would never allow to exist.

## 1.2  Why IPv6 Did Not Solve This

IPv6 solved the address exhaustion problem. It did not change the fundamental model: any host can still send packets to any reachable address. IPv6 nodes are still exposed by default. Services still require firewalls. Authentication and encryption are still optional. DDoS attacks work identically over IPv6. The attack surface is the same; the addresses are just longer.

The root cause is philosophical, not technical. IPv4 and IPv6 both treat the network as a neutral carrier of packets. Security is the responsibility of endpoints. ZTLP inverts this: the network refuses to carry packets that cannot prove their identity. Security becomes structural rather than bolted on.

## 1.3  Why Now

The prerequisites for a protocol like ZTLP did not exist a decade ago. They exist today:

- Hardware identity is mainstream. YubiKeys, TPM 2.0 chips, and Secure Enclaves are now standard in consumer and enterprise hardware. Hardware-bound private keys that cannot be extracted or cloned are no longer exotic or expensive.

- QUIC provides a proven encrypted UDP transport substrate. ZTLP does not need to solve NAT traversal, encrypted transport, or firewall compatibility from scratch. QUIC has already done that work, and it is now deployed at Internet scale.

- Zero Trust Architecture is now industry consensus. NIST SP 800-207, the US Executive Order on Improving the Nation’s Cybersecurity (2021), and widespread enterprise adoption have established Zero Trust as the direction of travel. ZTLP embeds this model at the network layer rather than requiring it to be retrofitted at every application.

- The Noise Protocol Framework provides a proven handshake foundation. WireGuard demonstrated that a simple, auditable, high-performance cryptographic protocol can achieve widespread real-world deployment. ZTLP builds on this foundation.

- The regulatory and threat environment demands action. CMMC 2.0, HIPAA enforcement, critical infrastructure protection mandates, and escalating cyber insurance requirements are creating strong economic incentives for organizations to move beyond perimeter-based security models. ZTLP aligns with and accelerates that direction.

## 1.4  Who Benefits First

ZTLP does not require global deployment to deliver value. The following sectors have acute pain points that ZTLP directly addresses, making them natural early adopters:

- Healthcare. Hospitals and clinics are the most targeted sector for ransomware. HIPAA mandates data protection but provides no network-layer enforcement mechanism. ZTLP makes lateral movement structurally impossible within a ZTLP-enrolled network: no open ports, no unauthenticated access, no attack surface for an attacker who gains a foothold on one device.

- Defense contractors and CMMC-regulated organizations. CMMC Level 2 and 3 requirements map directly to ZTLP capabilities: access control, identification and authentication, system and communications protection. ZTLP provides these at the network layer, reducing the compliance burden on individual applications.

- Managed Service Providers (MSPs). MSPs manage networks across dozens or hundreds of client environments, often connected via VPNs and jump hosts that are themselves attack targets. ZTLP replaces the VPN mesh with identity-based direct sessions, eliminates jump hosts as an attack surface, and makes client network isolation structural rather than policy-dependent.

- Manufacturing and industrial control systems. OT and ICS environments increasingly connect to IT networks and the Internet, creating attack paths into physical systems. ZTLP’s zero-open-port model provides a network layer that industrial devices can participate in without exposing control interfaces to the public Internet.

- Internet of Things deployments. IoT devices with hardware identity (TPM or Secure Enclave) can participate in ZTLP networks without any application-layer security configuration. The device either has a valid identity and is enrolled, or it cannot communicate with enrolled services. This eliminates entire categories of IoT attack.

## 1.5  The Vision

A network where ZTLP is widely deployed looks fundamentally different from the Internet of today:

- Port scanners return nothing. There are no open ports to find. Services do not exist to unenrolled nodes.

- DDoS floods exhaust bandwidth at the physical link but cannot exhaust CPU or memory — unauthenticated packets allocate no state.

- A stolen laptop or cloned VM cannot authenticate. The hardware token is the credential; without it the identity does not exist.

- Ransomware that gains a foothold on one device cannot move laterally. Every other service on the network requires a separate authenticated identity to reach.

- Network connectivity follows identity, not location. A device works identically whether it is in the office, at home, or on a cellular connection — because the network authenticates who you are, not where you are.

This is not a utopian vision. Every component described above already exists in some form. What does not yet exist is a unified, open protocol that assembles them into a coherent network layer. That is what ZTLP proposes to be.

# 2.  Introduction and Motivation

The Internet's current network layer (IPv4/IPv6) was designed with reachability as the primary objective. Security was not a first-class design principle. As a result, the global Internet suffers from persistent structural problems that cannot be fully resolved within the IPv4/IPv6 paradigm:

- Any host can send packets to any reachable address, regardless of authorization.

- Services are exposed by default; security is enforced reactively by firewalls and ACLs.

- DDoS attacks exploit the fact that routers cannot distinguish authorized from unauthorized traffic.

- BGP routing prioritizes policy over performance; path selection is opaque to endpoints.

- Encryption and authentication are optional add-ons, not protocol requirements.

- Identity is tied to IP addresses, which are location-based and easily spoofed.

ZTLP proposes a different model. Rather than replacing IPv6, ZTLP defines a secure overlay that rides on existing Internet infrastructure and enforces a single rule before any network state is allocated:

"If you cannot prove who you are, you do not exist on this network."

This produces a network layer where port scanning is useless, SYN floods allocate no state, spoofed packets are immediately discarded, and services have no externally visible attack surface unless explicitly permitted. DDoS mitigation becomes structural rather than reactive.

ZTLP is not a replacement for the Internet. It is a protected lane on top of it.

# 2.  Terminology

The key words MUST, MUST NOT, REQUIRED, SHALL, SHALL NOT, SHOULD, SHOULD NOT, RECOMMENDED, MAY, and OPTIONAL in this document are to be interpreted as described in RFC 2119.

| Term | Definition |
| --- | --- |
| ZTLP Node | Any device running the ZTLP stack with a valid Node Identity. |
| Node ID | A 128-bit identifier derived from the hash of the node's public key. |
| Service ID | A 128-bit identifier for a named service, derived from hash(service-name + tenant). |
| ZTLP Relay | A ZTLP Node that forwards traffic between other nodes across the public Internet. |
| ZTLP Gateway | A ZTLP Node that bridges between ZTLP and legacy IPv4/IPv6 networks. |
| ZTLP-NS | The ZTLP Namespace — a distributed DNS-like trust namespace for identity and service discovery. |
| Trust Anchor | A hardcoded public key embedded in ZTLP implementations that bootstraps initial trust. |
| Session | An established, mutually authenticated, encrypted ZTLP communication channel. |
| HeaderAuthTag | A 128-bit authentication tag over the ZTLP header allowing fast pre-decryption packet rejection. |
| Hardware Identity | A private key bound to a physical hardware token (e.g., YubiKey, TPM, Secure Enclave). |
| Policy Tag | A 32-bit compact field encoding tenant, role, or zone for fast edge policy decisions. |
| CryptoSuite | A 16-bit identifier specifying the AEAD, hash, and handshake algorithms in use. |

# 3.  Goals, Non-Goals, and Design Philosophy

## 3.1  Goals

- Enforce cryptographic authentication before allocating any network state.

- Eliminate the concept of 'open ports' — services are invisible without authorization.

- Support hardware-backed identity (YubiKey, TPM, Secure Enclave) as a first-class primitive.

- Provide performance-aware, multipath relay routing across the public Internet.

- Deploy incrementally over existing IPv4/IPv6 infrastructure without router changes.

- Define a distributed, DNS-like trust namespace (ZTLP-NS) for identity and service discovery.

- Support gradual migration from legacy networks (Phase 1 through Phase 4 model).

- Provide structural DDoS resistance through cryptographic air-gapping of the packet surface.

## 3.2  Non-Goals

- ZTLP does not propose a new IP version number or changes to IPv4/IPv6 headers.

- ZTLP does not require Internet routers or BGP to be modified.

- ZTLP v0.5 does not implement hard bandwidth reservations across the public Internet backbone. Bandwidth-aware path selection and soft admission control via the BANDWIDTH_HINT extension TLV are defined in Section 14. Hard per-session bandwidth reservation between participating relay operators is a target capability for a future revision — motivated primarily by its application to volumetric DDoS mitigation, where pre-allocated authenticated capacity lanes prevent attackers from consuming resources reserved for legitimate traffic.

- ZTLP is not a VPN product, though it shares architectural concepts with secure overlays.

- ZTLP does not provide anonymity — identity is a core requirement of the protocol.

## 3.3  Design Philosophy

Every design decision in ZTLP is evaluated against two governing principles. These are not aspirational — they are the criteria by which proposed changes to this specification MUST be judged.

### Principle 1 — DDoS Resistance Must Be Structural, Not Reactive

The current Internet responds to DDoS after the fact: detect the attack, classify the traffic, apply rate limits, absorb or block. This approach is expensive, slow, and fundamentally asymmetric — the attacker spends pennies per gigabit while the defender spends dollars. ZTLP inverts this entirely.

In ZTLP, a packet that cannot prove its identity costs the defender nothing to reject — a single header field check, no memory allocated, no state created, no cryptographic work performed. The cost asymmetry flips: the attacker must generate billions of packets and they accomplish nothing. The defender discards them for free.

Any feature added to ZTLP that increases the cost of processing unauthenticated packets — or that creates exploitable state before authentication — MUST be rejected. DDoS resistance is a hard architectural constraint, not a feature that can be traded off for convenience.

### Principle 2 — Identity Must Precede Communication

The current Internet allows any host to send packets to any reachable address. Security is the endpoint's problem. Firewalls, intrusion detection systems, WAFs, VPNs, and Zero Trust access proxies all exist to compensate for this single design choice. They are expensive, complex, and they fail regularly.

ZTLP moves this check to the network layer and makes it mandatory. A node without a cryptographically verified identity cannot initiate communication. A service without an enrolled identity cannot receive communication. There are no open ports. There is no visible attack surface. There is no unauthenticated state. The Zero Trust model is not enforced by policy on top of an insecure network — it is enforced by the protocol itself, at the packet level, before anything else happens.

These two principles — structural DDoS resistance and mandatory identity — are not independent. They are the same idea expressed at different layers. An unauthenticated packet is both a potential DDoS vector and a Zero Trust violation. ZTLP treats them as one problem with one solution: if you cannot prove who you are, you do not exist on this network.

# 4.  Threat Model

ZTLP is designed to mitigate the following classes of threat:

| Threat | Description | ZTLP Mitigation |
| --- | --- | --- |
| Volumetric DDoS | Flood of packets to exhaust CPU, memory, or bandwidth. | HeaderAuthTag validation rejects unauthenticated packets in microseconds; no state allocated. |
| Packet Spoofing | Attacker forges source address. | Every packet carries a HeaderAuthTag tied to session keys; forgeries fail immediately. |
| Port Scanning | Attacker enumerates open services. | No services are visible; ZTLP nodes do not respond to unauthenticated probes. |
| Replay Attacks | Attacker replays captured valid packets. | PacketSeq (64-bit) and Timestamp fields enforce anti-replay windows. |
| Route Hijacking | BGP prefix hijack redirects traffic. | ZTLP traffic is encrypted and authenticated end-to-end; hijacked routes carry opaque ciphertext. |
| Man-in-the-Middle | Attacker intercepts and modifies traffic. | Mutual authentication during handshake; session keys are ephemeral and hardware-bound. |
| Credential Theft | Private key or certificate stolen. | Hardware-bound keys (YubiKey/TPM) cannot be extracted; short-lived session tokens limit exposure. |
| Lateral Movement | Compromised node attacks others internally. | Each service requires separate policy authorization; Node ID alone does not grant access. |
| Identity Cloning | VM clone or config copy impersonates a node. | Hardware-bound keys mean a cloned config without the physical hardware token cannot authenticate. |

ZTLP does NOT protect against:

- Bandwidth saturation at the physical link layer before packets reach ZTLP edges.

- Compromise of the hardware identity device itself.

- Application-layer attacks from legitimately authenticated nodes.

# 5.  Protocol Overview

ZTLP operates as a secure overlay. The public Internet (IPv4/IPv6) acts as a dumb transport substrate. ZTLP does not require routers or ISPs to understand the protocol.

A ZTLP packet is an encrypted, authenticated payload carried inside a standard UDP or QUIC datagram. To the public Internet, ZTLP traffic appears as ordinary encrypted UDP — indistinguishable from QUIC, DNS-over-QUIC, or other encrypted UDP traffic.

## 5.1  Layering

[ Physical / Ethernet ]

[ IPv4 or IPv6 ]           ← public Internet routing

[ UDP or QUIC ]            ← transport substrate

[ ZTLP Header ]            ← identity, auth, routing

[ Encrypted Payload ]      ← application data

## 5.2  Connection Model

ZTLP uses a connection-oriented model with explicit session establishment. The phases are:

- Node discovers relay via bootstrap procedure (Section 9).

- Node performs HELLO handshake with relay or peer (Section 10).

- Mutual identity verification and policy check.

- Session keys derived; short-lived session established.

- Data packets flow; keys rotate automatically per session policy.

- Session closes via explicit CLOSE message or timeout.

## 5.3  Identity vs. Location

In IPv4/IPv6, an address is a location. In ZTLP, identity is primary. A Node ID is a hash of the node's public key — it is the same regardless of what IPv6 address the node currently uses. This enables seamless mobility, multihoming, and relay-based routing without breaking sessions.

# 6.  ZTLP Addressing — Node Identity

## 6.1  Node ID

A ZTLP Node ID is a 128-bit value derived as:

NodeID = TRUNCATE_128(SHA3-256(public_key))

Node IDs are self-certifying: any party that knows a public key can independently compute and verify the associated Node ID. Node IDs are NOT routable addresses. They are identity handles used for authentication and policy decisions.

## 6.2  Service ID

Services are identified independently from the hosts that run them:

ServiceID = TRUNCATE_128(SHA3-256(service-name || tenant-id))

Example Service IDs resolve from human-readable names in ZTLP-NS:

rdp.clinic.acmedental.ztlp  →  ServiceID: 8f23:9c11:ae45:...

backup.server.example.ztlp  →  ServiceID: c341:0f8a:21bc:...

## 6.3  Human-Readable Names

The ZTLP-NS namespace (Section 8) maps human-readable names to Node IDs, Service IDs, relay addresses, and policy records — functioning similarly to DNS but with mandatory signing and delegated trust.

# 7.  ZTLP Packet Format

## 7.1  Base Header (64 bytes, fixed)

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Magic (8)    |Ver(4)|HdrLen |    Flags (16)  |  MsgType (8) |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   CryptoSuite (16)            |   KeyID / TokenID (16)       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     SessionID (64 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     PacketSeq (64 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     Timestamp (64 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     SrcNodeID (128 bits)                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     DstSvcID  (128 bits)                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  PolicyTag (32)               | ExtLen (16)  | PayloadLen(16)|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   HeaderAuthTag (128 bits)                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

## 7.2  Field Definitions

| Field | Size | Description |
| --- | --- | --- |
| Magic | 8 bits | Fixed value 0x5A37 ('Z7'). Allows fast rejection of random UDP noise before any parsing. |
| Ver | 4 bits | Protocol version. Current: 1. |
| HdrLen | 12 bits | Header length in 4-byte words, including extensions. |
| Flags | 16 bits | Bitfield: HAS_EXT, ACK_REQ, REKEY, MIGRATE, MULTIPATH, RELAY_HOP. |
| MsgType | 8 bits | DATA, HELLO, HELLO_ACK, REKEY, CLOSE, ERROR, PING, PONG. |
| CryptoSuite | 16 bits | Identifies AEAD + hash + handshake family (e.g., ChaCha20-Poly1305 + Noise_XX). |
| KeyID/TokenID | 16 bits | Selects active credential slot; allows edge to pick correct verification key instantly. |
| SessionID | 64 bits | Stable per-flow identifier. Assigned during HELLO. Rotates on REKEY. |
| PacketSeq | 64 bits | Monotonically increasing per-session counter. Anti-replay window enforced. |
| Timestamp | 64 bits | Unix epoch milliseconds. Packets outside ±30 seconds of receiver clock MUST be rejected. |
| SrcNodeID | 128 bits | Sender's Node ID (hash of public key). Zero during initial HELLO. |
| DstSvcID | 128 bits | Destination Service ID. No port numbers; services are resolved by identity. |
| PolicyTag | 32 bits | Compact policy hint (tenant/role/zone). Edge uses for fast admission decisions. |
| ExtLen | 16 bits | Length in bytes of extension TLV area following the base header. |
| PayloadLen | 16 bits | Length in bytes of the encrypted payload following extensions. |
| HeaderAuthTag | 128 bits | AEAD tag over the base header. Verified BEFORE decrypting payload. Invalid = silent drop. |

## 7.3  Extension TLVs (Optional)

If HAS_EXT flag is set, a Type-Length-Value extension area follows the base header:

| Type | Name | Description |
| --- | --- | --- |
| 0x01 | PATH_HINT | Preferred relay node IDs for this flow. |
| 0x02 | DEVICE_POSTURE | Signed attestation blob from TPM/Secure Enclave. |
| 0x03 | ROUTE_SCOPE | Site or realm identifier for routing policy. |
| 0x04 | TRACE_CTX | Correlation ID for distributed tracing and diagnostics. |
| 0x05 | BANDWIDTH_HINT | Requested bandwidth profile (informational; not a hard reservation). |
| 0x06 | RELAY_PATH | Ordered list of relay Node IDs traversed (for diagnostics). |

## 7.4  Payload

The payload is AEAD-encrypted application data. The authentication tag produced by the AEAD cipher is appended to the ciphertext. Decryption MUST fail and the packet MUST be silently dropped if authentication fails.

# 8.  ZTLP-NS — Distributed Trust Namespace

ZTLP-NS is the control-plane identity and discovery layer. It is inspired by DNS delegation and DNSSEC chain-of-trust principles, adapted for ZTLP's identity-first model.

## 8.1  Design Principles

- Hierarchical delegation — trust flows from root anchors through operator zones to individual nodes.

- All records are signed — unsigned records MUST be rejected.

- Federated roots — multiple trust roots are supported; no single global authority.

- ZTLP-NS is a control-plane lookup layer — it is NOT used for per-packet routing decisions.

## 8.2  Namespace Structure

Root Trust Anchors (embedded in ZTLP software)

└── Operator Zones  (e.g., example.ztlp)

└── Tenant Zones  (e.g., acmedental.example.ztlp)

└── Node / Service Records

## 8.3  Record Types

| Record Type | Description | Example |
| --- | --- | --- |
| ZTLP_KEY | Node's public key and Node ID. | node1.office.acmedental.ztlp → NodeID + pubkey |
| ZTLP_SVC | Service definition: ServiceID, allowed Node IDs, policy. | rdp.acmedental.ztlp → ServiceID + policy |
| ZTLP_RELAY | Relay node: Node ID, IPv6 endpoints, capacity metrics. | relay1.apac.ztlp → NodeID + endpoints |
| ZTLP_POLICY | Access policy: which Node IDs can reach which Service IDs. | policy.acmedental.ztlp → ACL |
| ZTLP_REVOKE | Revocation notice for a Node ID or Token ID. | revoke.acmedental.ztlp → revoked IDs + timestamp |
| ZTLP_BOOTSTRAP | Signed list of relay nodes for initial discovery. | bootstrap.ztlp → signed relay list |

## 8.4  Federated Trust Roots

ZTLP-NS supports multiple trust roots for different deployment contexts. Implementations SHOULD include at least the following root categories:

- Public ZTLP Root — maintained by the ZTLP protocol governance body.

- Enterprise Root — self-hosted by organizations for private deployments.

- Industry Roots — sector-specific (e.g., healthcare, government, finance).

A node MAY trust multiple roots simultaneously. Policy records specify which trust roots are accepted for a given service.

# 9.  Node Initialization and Bootstrap Procedure

A ZTLP node that has no prior session state MUST follow the Node Initialization Procedure (NIP) to discover its first relay connection. This procedure is defined normatively.

## 9.1  Bootstrap Sequence

The following steps MUST be attempted in order. A node MUST proceed to the next step only if the current step fails or returns no usable result.

### Step 1 — HTTPS Discovery (REQUIRED)

The node MUST attempt an HTTPS GET request to each URL in the hardcoded discovery URL list. The response MUST be a signed JSON object containing:

- A list of relay node addresses (IPv6 and IPv4).

- Each relay's Node ID and public key.

- A validity timestamp and TTL.

- A signature verifiable against the software trust anchor.

The node MUST validate the signature before using any relay address from the response. The node MUST cache a valid response locally with its stated TTL, minimum 24 hours.

HTTPS is chosen as the primary mechanism because it survives NAT, CGNAT, and enterprise firewalls in nearly all deployment environments.

### Step 2 — DNS-SRV Discovery (RECOMMENDED)

The node SHOULD query for SRV records at the well-known bootstrap domain:

_ztlprelay._udp.bootstrap.ztlp

If DNSSEC validation is available, the node MUST validate the response chain before use. If validation fails, the node MUST treat the response as untrusted and proceed to Step 3.

### Step 3 — Hardcoded Trust Anchors (FALLBACK)

If Steps 1 and 2 both fail, the node MUST attempt connection to the hardcoded relay node list embedded in the software distribution. This list:

- MUST contain a minimum of 15 relay nodes.

- MUST span a minimum of 5 distinct Autonomous Systems (ASNs).

- MUST span a minimum of 3 geographic regions.

- MUST include the pre-pinned public key for each relay.

The hardcoded list is a bootstrap-of-last-resort. Nodes that successfully complete Step 1 SHOULD NOT rely on hardcoded anchors for subsequent connections.

## 9.2  Post-Bootstrap Peer Exchange

Once a node establishes a session with any relay, it MUST request the relay's routing table via a PEER_EXCHANGE message. The relay responds with a signed list of known relay nodes. The node caches this list and uses it for all future connections, independent of the bootstrap anchors.

## 9.3  Discovery URL Resilience

Implementations MUST include discovery URLs hosted by at least 3 independent operators. This prevents a single-point-of-failure in HTTPS discovery. Discovery URL operators MUST be geographically and organizationally independent.

# 10. Handshake and Session Establishment

## 10.1  Handshake Overview

ZTLP uses a Noise Protocol Framework pattern (Noise_XX) for session establishment. This provides:

- Mutual authentication — both parties prove identity.

- Perfect forward secrecy — compromise of long-term keys does not expose past sessions.

- Identity hiding — identities are encrypted after the first message.

## 10.2  Message Flow

Initiator                          Responder

|                                   |

|--- HELLO (ephemeral pubkey) -----> |

|                                   |

|<-- HELLO_ACK (ephemeral pubkey,   |

|    encrypted identity + cert) ---- |

|                                   |

|--- (encrypted identity + cert,    |

|    policy request) -------------> |

|                                   |

|<-- SESSION_OPEN (SessionID,       |

|    PolicyTag, session keys) ------ |

|                                   |

|=== DATA flows (encrypted) ======= |

## 10.3  Policy Enforcement

The Responder MUST perform the following checks before issuing SESSION_OPEN:

- Verify the Initiator's certificate chain against a trusted ZTLP-NS root.

- Verify the certificate has not been revoked (ZTLP_REVOKE record check).

- Evaluate policy: is SrcNodeID authorized to reach DstSvcID?

- Check device posture if DEVICE_POSTURE extension is present and required by policy.

- Reject with ERROR if any check fails. No state is allocated on failure.

## 10.4  Key Rotation

Session keys MUST be rotated at intervals defined by the CryptoSuite policy, or upon explicit REKEY message. A new SessionID is issued on each rotation. Applications MUST NOT experience connection interruption during key rotation.

# 11. Relay Node Architecture

## 11.1  Role of Relay Nodes

ZTLP Relay nodes form the backbone of the overlay network. They perform three functions:

- Forward authenticated ZTLP traffic between nodes that cannot communicate directly.

- Participate in peer exchange, distributing routing table information.

- Publish capacity metrics (latency, packet loss, bandwidth) to the ZTLP-NS.

## 11.2  Relay Selection Criteria

Nodes MUST select relays using a path score computed from:

PathScore = w1*Latency + w2*PacketLoss + w3*Congestion + w4*TrustScore

Lower PathScore is preferred. Weights (w1–w4) are configurable per deployment. Nodes MUST maintain at least 2 active relay paths simultaneously for failover.

## 11.3  Relay Operator Requirements

Relay nodes MUST:

- Maintain a valid ZTLP_RELAY record in at least one ZTLP-NS zone.

- Publish latency and capacity metrics no less than once per 60 seconds.

- Support all four transport modes defined in Section 12.

- Honor CLOSE messages and release session state within 30 seconds.

- Not log plaintext payload content (relays see only encrypted data).

# 12. Hardware Enforcement Profiles

The ZTLP base specification assumes packet enforcement occurs at ZTLP-aware endpoint software. This is sufficient for most deployments but leaves a gap at Layer 1 and Layer 2: volumetric attack traffic still reaches the ZTLP edge node and saturates the physical uplink before the HeaderAuthTag check can discard it.

This section defines four Hardware Enforcement Profiles that push ZTLP enforcement progressively closer to the wire. Each profile is independently implementable. Profiles are additive — a deployment MAY implement multiple profiles simultaneously. All profiles feed enforcement decisions from the ZTLP control plane; no profile requires the switch or NIC to independently validate full cryptographic identity.

## 12.1  Profile 1 — Software Enforcement (eBPF/XDP)

The software profile is the baseline enforcement model and is implementable today on any Linux host using eBPF and XDP (eXpress Data Path). XDP programs attach directly to the NIC driver and execute before the Linux kernel network stack, allowing packet decisions at near line-rate in software.

### 12.1.1  Enforcement Logic

An XDP program on a ZTLP node MUST implement the following decision pipeline for every inbound UDP packet on the ZTLP port:

1. Read first 8 bits. If Magic != 0x5A37 → XDP_DROP (immediate, zero state)

2. Read SessionID field. Check against eBPF SessionID allowlist map.

If SessionID not in map AND MsgType != HELLO → XDP_DROP

3. Rate-limit HELLO packets per source IP (token bucket in eBPF map).

If rate exceeded → XDP_DROP

4. Pass to kernel → full HeaderAuthTag verification in ZTLP userspace daemon.

The SessionID allowlist is a BPF hash map maintained by the ZTLP daemon. When a session is established, the daemon writes the SessionID to the map. When a session closes or expires, it is removed. The XDP program reads this map without kernel involvement — lookup time is O(1) at nanosecond scale.

### 12.1.2  What This Achieves

- Random UDP noise and Magic mismatches are dropped at the NIC driver before consuming any kernel CPU.

- Unknown SessionIDs (spoofed or replayed packets) are dropped before reaching the ZTLP daemon.

- HELLO flood attacks (handshake exhaustion) are rate-limited per source IP before any cryptographic work is performed.

- Sustained attack traffic of tens of millions of packets per second can be absorbed on a single modern host without service degradation.

### 12.1.3  Deployment Requirements

- Linux kernel 5.4 or later (XDP support required).

- NIC with XDP native driver support (Intel i40e, Mellanox mlx5, or equivalent).

- ZTLP daemon responsible for maintaining the SessionID BPF map in real time.

## 12.2  Profile 2 — SmartNIC Offload (DPU)

Data Processing Units (DPUs) — also called SmartNICs — are network interface cards with dedicated ARM or RISC-V processors capable of running arbitrary packet processing logic independently of the host CPU. Representative hardware includes the Nvidia BlueField-3 DPU and the AMD Pensando DSP. This profile moves full ZTLP HeaderAuthTag verification off the host CPU and onto the DPU, so invalid packets are rejected in hardware before consuming any host resources.

### 12.2.1  Enforcement Architecture

The ZTLP daemon on the host synchronizes session state to the DPU via a shared control channel. The DPU runs a ZTLP Offload Agent that maintains:

- Session key table: SessionID mapped to the AEAD session key, loaded from host daemon on session establishment.

- Anti-replay window: Per-session PacketSeq bitmap maintained on the DPU for replay rejection.

- HELLO rate limiter: Per-source-IP token bucket enforced before any handshake packet reaches the host.

For every inbound packet, the DPU Offload Agent performs the full ZTLP verification pipeline — Magic check, SessionID lookup, HeaderAuthTag AEAD verification, PacketSeq anti-replay — entirely on the DPU. Only packets that pass all checks are forwarded to the host PCIe bus. Attack traffic never reaches host memory.

### 12.2.2  Security Properties

- Full cryptographic HeaderAuthTag verification occurs at line rate without host CPU involvement.

- A compromised host OS cannot bypass DPU enforcement — the DPU is a separate trust boundary.

- The DPU can store session keys in its own secure memory, isolated from host RAM — preventing key extraction even under a host kernel compromise.

- This profile is RECOMMENDED for ZTLP relay nodes and gateway nodes handling high-volume traffic.

## 12.3  Profile 3 — Programmable Switch Enforcement (P4)

P4 is a domain-specific language for programming the forwarding behavior of network device ASICs — including switches, routers, and FPGAs. P4-capable switch hardware (Intel Tofino, Nvidia Spectrum-3 with P4, Barefoot Networks) can be programmed to inspect arbitrary packet fields and make forwarding decisions at full line rate — hundreds of millions of packets per second — with deterministic nanosecond latency.

This profile defines a ZTLP P4 program that enforces packet admission at the switch ASIC, before traffic reaches any server. This is the closest currently achievable approximation to making a network switch natively enforce ZTLP.

### 12.3.1  What P4 Can Enforce at Line Rate

Switch ASICs cannot perform AEAD cryptographic operations at line rate — this is a hardware constraint that applies to all current programmable switch silicon. The P4 profile therefore splits enforcement into two tiers:

- Tier 1 — Switch ASIC (P4, line rate): Magic byte check, SessionID allowlist lookup, source IP rate limiting, HELLO packet metering. Packets failing Tier 1 are dropped at the switch with no server involvement whatsoever.

- Tier 2 — Server (eBPF or DPU): Full HeaderAuthTag AEAD verification, anti-replay window enforcement, policy check. Only packets that survived Tier 1 reach this layer, so the server processes a dramatically reduced volume of packets under attack conditions.

### 12.3.2  ZTLP Control Plane to Switch Integration

The ZTLP control plane populates the switch’s P4 match-action tables via a southbound API (P4Runtime over gRPC). Session lifecycle events trigger table updates:

SESSION_OPEN  → write SessionID to P4 allowlist table

SESSION_CLOSE → remove SessionID from P4 allowlist table

REKEY         → update SessionID entry with new session identifier

REVOKE        → immediately flush all entries for revoked NodeID

This integration means the switch’s allowlist is always consistent with the ZTLP control plane. A revoked identity is removed from the switch table within one control plane propagation cycle — typically under one second — after which the switch drops all traffic from that identity at the ASIC level, with no server involvement.

### 12.3.3  Deployment Requirements

- P4-capable switch hardware with P4Runtime API support.

- ZTLP control plane with P4Runtime southbound interface to the switch.

- Profile 1 or Profile 2 enforcement at the server for Tier 2 cryptographic verification.

## 12.4  Profile 4 — Native Switch ASIC (Future)

This profile describes the long-term target state: switch ASICs that natively understand ZTLP packet structure and enforce admission decisions in hardware without programming via P4. This is analogous to how modern switches natively implement MACsec (IEEE 802.1AE) — a Layer 2 encryption and authentication standard that switch silicon enforces transparently without external programming.

### 12.4.1  ZTLP-Switch Minimum Feature Set

A switch claiming ZTLP-Native compliance MUST implement the following in ASIC hardware:

- ZTLP Magic byte recognition and hard drop on mismatch.

- Hardware SessionID allowlist table with control plane API for population and revocation.

- Per-source-IP rate limiting for MsgType HELLO at configurable thresholds.

- ZTLP control plane southbound interface (ZTLP-CP API) for session table management.

- Drop counters per enforcement rule, exportable via standard telemetry (gNMI/OpenConfig).

Full on-chip HeaderAuthTag AEAD verification is listed as an OPTIONAL capability for Profile 4 hardware, recognizing that AES-GCM and ChaCha20-Poly1305 acceleration in switch silicon is an emerging capability. Switches implementing on-chip AEAD MAY bypass server-side Tier 2 verification for established sessions.

### 12.4.2  Relationship to MACsec (IEEE 802.1AE)

MACsec provides hop-by-hop Layer 2 encryption and authentication between directly connected devices. It is widely implemented in enterprise switch ASICs and is the best existing precedent for hardware-enforced cryptographic authentication at the network layer. ZTLP Profile 4 is conceptually similar to MACsec but operates at Layer 3/4 across arbitrary network paths rather than between adjacent Layer 2 peers. Switch vendors familiar with MACsec implementation will find the ZTLP-Native hardware requirements structurally similar in scope.

## 12.5  Profile Comparison Summary

| Profile | Enforcement Point | Full AuthTag? | Available Today? | Host CPU Used? | HW Cost |
| --- | --- | --- | --- | --- | --- |
| 1 — eBPF/XDP | NIC driver (software) | Daemon only | Yes | Minimal (XDP) | Low ($0 extra) |
| 2 — SmartNIC/DPU | DPU (hardware) | Yes (on DPU) | Yes | None (DPU handles) | Medium (DPU card) |
| 3 — P4 Switch | Switch ASIC | Tier 1 only (SessionID) | Yes (P4 hardware) | Tier 2 only | High (P4 switch) |
| 4 — Native ASIC | Switch silicon | Full (optional AEAD) | Future (vendor req.) | None | Standard switch |

# 13. Transport Fallback and NAT Traversal

ZTLP MUST function in environments where ISPs, enterprise firewalls, or carrier-grade NAT devices block or reshape UDP traffic on non-standard ports. Implementations MUST support the following transport fallback ladder in order:

| Priority | Transport | Condition | Notes |
| --- | --- | --- | --- |
| 1 | UDP / ZTLP Port | Preferred | Native ZTLP transport. Best performance. |
| 2 | UDP / 443 | If port blocked | Harder for ISPs to block without disrupting QUIC. |
| 3 | TCP / 443 (TLS framed) | If UDP blocked | ZTLP framed inside TLS record layer. |
| 4 | WebSocket over HTTPS | Last resort | Maximum firewall compatibility. Higher overhead. |

Relay nodes MUST support all four transport modes. Initiating nodes MUST attempt them in order and use the first that succeeds. The selected transport SHOULD be remembered and used for subsequent connections to the same relay.

## 12.1  NAT Traversal

For direct peer-to-peer connections (without relay), ZTLP nodes SHOULD attempt UDP hole-punching coordinated via a relay node. If hole-punching fails within 5 seconds, the relay MUST continue to forward the session.

# 14. Routing and Path Selection

ZTLP does not modify BGP or Internet routing. Path selection occurs within the ZTLP overlay, using relay-published metrics.

## 13.1  Multipath

ZTLP nodes MAY split flows across multiple relay paths. This provides:

- Resilience against relay outages.

- Aggregate bandwidth across paths.

- Geographic traffic distribution.

The MULTIPATH flag in the header signals that a session uses multipath mode. Receivers MUST reassemble using PacketSeq.

## 13.2  Mobility

Because Node IDs are identity-based and not location-based, a node that changes its underlying IPv6 address (e.g., Wi-Fi to cellular) MUST issue a MIGRATE message containing its new transport endpoint. The existing session continues without interruption. Applications MUST NOT observe a connection break during migration.

# 15. Key Management and Hardware Identity

## 14.1  Hardware-Backed Keys

ZTLP STRONGLY RECOMMENDS that long-term private keys be stored in hardware security devices. Supported hardware includes:

- FIDO2 / YubiKey (USB, NFC)

- Trusted Platform Module (TPM 2.0)

- Secure Enclave (Apple Silicon, ARM TrustZone)

- Hardware Security Module (HSM) for relay and gateway nodes

When hardware identity is in use, the private key MUST NOT be exportable. Signing operations MUST occur on-device. A cloned VM or stolen configuration file WITHOUT the physical hardware token MUST NOT be able to authenticate as the original node.

## 14.2  Certificate Lifecycle

Every ZTLP node MUST have a certificate issued by a ZTLP-NS trust root. The certificate lifecycle is:

- Enrollment — node generates key pair on hardware; submits public key to ZTLP-NS operator.

- Issuance — operator validates identity; issues signed certificate with TTL.

- Renewal — certificate renewed before expiry; hardware must sign the renewal request.

- Revocation — operator publishes ZTLP_REVOKE record; nodes MUST check revocation.

Certificate TTL SHOULD NOT exceed 90 days. Short-lived session tokens (issued per-session) SHOULD NOT exceed 24 hours.

# 16. Deployment Model and Migration Strategy

ZTLP is designed for incremental deployment. Organizations can adopt ZTLP in phases without disrupting existing infrastructure.

| Phase | Description | Legacy Compatibility |
| --- | --- | --- |
| Phase 1 — Overlay | ZTLP nodes communicate with each other over the overlay. Legacy IPv4/IPv6 traffic is unchanged. | Full — ZTLP is additive. |
| Phase 2 — Gateway | ZTLP Gateways bridge between ZTLP overlay and legacy systems. Legacy clients can reach ZTLP services via gateway. | High — legacy clients need no changes. |
| Phase 3 — Preferred | ZTLP paths are preferred for sensitive services. Critical systems require ZTLP authentication. | Medium — legacy clients use gateway. |
| Phase 4 — Native | ZTLP is the default for all inter-node communication. Legacy fallback is restricted or deprecated. | Low — legacy clients require gateway or upgrade. |

## 15.1  Gateway Operation

A ZTLP Gateway translates between ZTLP-authenticated sessions and legacy TCP/UDP connections. The gateway:

- Terminates the ZTLP session on behalf of the legacy service.

- Enforces ZTLP policy at the boundary.

- Hides the legacy service's real IP address from the public Internet.

- Logs session metadata (Node IDs, Service IDs, timestamps) for audit.

# 17. Security Considerations

## 16.1  DDoS Resistance

ZTLP's primary DDoS defense is structural: unauthenticated packets are rejected at the HeaderAuthTag check before any session state is allocated. This does not prevent link-layer saturation, but it eliminates CPU and memory exhaustion attacks that exploit state allocation.

## 16.2  Clock Synchronization

The Timestamp field requires nodes to have reasonably synchronized clocks. Nodes MUST reject packets with timestamps more than ±30 seconds from local time. Implementations SHOULD use NTP or PTP for clock synchronization.

## 16.3  Cryptographic Agility

The CryptoSuite field allows future algorithm upgrades without protocol version changes. Implementations MUST NOT negotiate CryptoSuites with known weaknesses. A registry of approved CryptoSuite identifiers will be maintained separately.

## 16.4  Revocation Latency

ZTLP_REVOKE records propagate through ZTLP-NS with TTL-governed latency. Implementations MUST check revocation status during session establishment. For high-security deployments, short certificate TTLs (hours rather than days) are preferred over reliance on revocation propagation.

# 18. Privacy Considerations

ZTLP's identity-first model introduces privacy tradeoffs relative to the anonymous packet model of IPv4/IPv6.

- Node IDs are persistent and could enable long-term traffic correlation.

- ZTLP-NS lookups for a node's services may reveal organizational relationships.

Mitigations:

- Rotating pseudonymous Node IDs for non-critical sessions (at cost of breaking session continuity).

- Ephemeral SessionIDs (rotated on REKEY) limit per-session correlation.

- ZTLP-NS operators SHOULD NOT log individual lookup queries.

This document acknowledges that ZTLP is not an anonymity protocol. Applications requiring anonymity SHOULD use additional layers (e.g., onion routing) above ZTLP.

# 19. Operational Tooling

Network operators MUST have access to diagnostic tools. The following tools are defined as part of the ZTLP operational suite:

| Tool | Function | Equivalent |
| --- | --- | --- |
| ztlp-ping | Test reachability to a Node ID or Service ID. | ping |
| ztlp-trace | Display relay path taken to a destination. | traceroute |
| ztlp-status | Show local node state, active sessions, current relay. | netstat |
| ztlp-relay-info | Query a relay node's metrics and peer table. | BGP show route |
| ztlp-ns-lookup | Resolve ZTLP-NS records for a name. | dig / nslookup |
| ztlp-revoke-check | Verify revocation status of a Node ID. | OCSP check |

# 20. Open Issues and Future Work

The following issues are known and will be addressed in subsequent draft revisions:

- Bandwidth reservation model — formal definition of the BANDWIDTH_HINT TLV semantics and interaction with relay QoS policies.

- Congestion control — specification of a ZTLP-native congestion control algorithm for long-haul relay paths.

- ZTLP-NS governance — formal governance structure for the public ZTLP-NS root.

- Relay operator incentives — economic models for relay node operation at scale.

- Lawful intercept considerations — engagement with regulatory frameworks.

- Hardware identity enrollment UX — simplifying the YubiKey/TPM enrollment flow for non-technical users.

- ZTLP for IoT — lightweight profile for constrained devices.

- Formal security proof — cryptographic analysis of the Noise_XX handshake under ZTLP's threat model.

# 21. References

## 20.1  Normative References

- RFC 2119 — Key words for use in RFCs to indicate requirement levels.

- RFC 8446 — TLS 1.3.

- RFC 7748 — Elliptic Curves for Diffie-Hellman Key Agreement (X25519, X448).

- Noise Protocol Framework — Trevor Perrin, 2018.

- RFC 9000 — QUIC: A UDP-Based Multiplexed and Secure Transport.

## 20.2  Informative References

- RFC 7401 — Host Identity Protocol Version 2 (HIPv2).

- RFC 6698 — The DANE Transport Layer Security (TLS) Protocol.

- SCION: A Secure Internet Architecture — Barrera et al., ETH Zurich.

- BeyondCorp: A New Approach to Enterprise Security — Ward & Beyer, Google.

- RFC 4960 — Stream Control Transmission Protocol (SCTP).

- RFC 8555 — Automatic Certificate Management Environment (ACME).

─────────────────────────────────────────

End of ZTLP Draft RFC v0.5

ZTLP.org — 2026